Electric Recovery Winch

Thank you for purchasing a  Winch. This manual covers operation and maintenance of the winch. All information in this publication is based on the latest production information available at the time of printing.

# General Safety Precautions

AWinch is designed to give safe and dependable service if operated according to the instructions. Read and understand this manual before installation and operation of winch.

Follow these general safety precautions:

- Don't use unsuitable pulleys or accessories.

- Don't use unsuitable rope in construction, strength or having any defects.

- Check the winch for smooth operation without load before winching operation.

- Make sure the wire rope is wound evenly on the first layer on the drum, rewind it if not

evenly wound.

WARNING

1. The winch is rated for intermittent-periodic duty. 2. The winch is not to be used to lift, support or otherwise transport personnel. 3. A minimum of five (5) wraps of rope around the drum are necessary to support the rated load. 4. When choosing the right winch, you need to consider the vehicle size and weight. As a general guide, you need a winch with a maximum load rating of at least one to one and a half times greater than the gross vehicle weight. 5. The rated line pull of the winch must be powerful enough to overcome the added resistance caused by whatever the vehicle is stuck in.

#### I. Safety Precautions

Please read and understand this Instruction Manual before installing your winch.

Don’t use unsuitable rope in construction, strength or having any defects.

Don’t use unsuitable hook and pulley block for rope.

The operator of winch in some cases, is required to have qualifications according to

applicable laws and ordinances.

Do not use winch as a lifting device or a hoist for

vertical lifting (Fig1).

Do not use winch to move people.

Do not exceed maximum line pull ratings shown in tables. Shock load must not exceed these ratings.

(Fig1)

Keep hands clear of wire rope and fairlead opening.

Pull from an angle below 15 degree to straighten up

the vehicle or load. (Fig2)

Use leather gloves or a heavy rag when handling the

wire rope.

Centre line of anchor point

When winching a heavy load lay a heavy blanket or

jacket over the wire rope near the hook end.

(Fig2)

#### II. Performance Data

Specifications

| Model |  |  | DV-6000 | DV-6000i | DV-9000 | DV-9000i | DV-12000 |  | DV-15000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Line Pull (first layer) |  |  | 2,722kg / 6,000lb |  | 4,082kg / 9,000lb |  | 5,443kg / 12,000lb |  | 6,803kg / 15,000lb |
| Line Speed (first layer, no load) |  |  | 14.6mpm / 48 fpm |  | 13.4mpm / 44 fpm |  | 8.3mpm / 27fpm |  | 6.9mpm / 22fpm |
| Amp. Draw | 12V |  | 400A 12 volt only |  | 350A |  | 440A |  | 460A |
|  | 24V |  |  |  | 210A |  | 290A |  | 340A |
| Motor | Type |  | Series wound |  |  |  |  |  |  |
|  | Output | 12V | 2,685 w / 3.6 hp 12 volt only |  | 3,430 w / 4.6 hp |  | 4,175 w / 5.6 hp |  |  |
|  |  | 24V |  |  | 1,940 w / 2.6 hp |  | 2,684 w / 3.6 hp |  |  |
| Gear Train | Type |  | 3 stage planetary |  |  |  |  |  |  |
|  | Ratio |  | 156:1 |  | 156:1 |  | 261:1 |  | 315:1 |
| Clutch |  |  | Rotating ring gear clutch |  |  |  |  |  |  |
| Brake |  |  | U.S. Patent cone load holding brake |  |  |  |  |  |  |
| Control |  |  | Detachable solenoid pack | Integrated solenoid pack | Detachable solenoid pack | Integrated solenoid pack | Detachable solenoid |  |  |
| Wire Rope | Type |  | A7 x 19Aircraft galvanized |  |  |  |  | 6 x W(19) |  |
|  | Length |  | 24.4m / 80ft |  | 30m / 100ft |  | 38m / 125 ft | 27m / 89 ft |  |
|  | Size |  | 8mm / 5/16in |  | 8mm / 5/16in |  | 9.5mm / 3/8 in | 11mm / 7/16 in |  |

Performance

| Model |  | DV-6000 | DV-6000i | DV-9000 | DV-9000i | DV-12000 | DV-15000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1st layer | Line pull (kg / lb) | 2,722 / 6,000 |  | 4,082 / 9,000 |  | 5,443 / 12,000 | 6,803 / 15,000 |
|  | Line speed (mpm / fpm) | 3.1 / 10 |  | 2.0 / 6.6 |  | 1.2 / 4 | 0.76 / 2.5 |
|  | Rope cap (m / ft) | 6.1 / 20 |  | 6.1 / 20 |  | 6.2 / 20.3 | 5.3 / 17.4 |
| 2nd layer | Line pull (kg / lb) | 2,224 / 4,903 |  | 3,335 / 7,352 |  | 4,561 / 10,056 | 5,575 / 12,292 |
|  | Line speed (mpm / fpm) | 3.7 / 12.1 |  | 2.4 / 7.9 |  | 1.43 / 4.7 | 0.92 / 3.01 |
|  | Rope cap (m / ft) | 13.5 / 44.3 |  | 13.5 / 44.3 |  | 13.6 / 44.6 | 11.8 / 38.7 |
| 3rd layer | Line pull (kg / lb) | 1,880 / 4,144 |  | 2,820 /6,217 |  | 3,926 / 8,657 | 4,723 / 10,413 |
|  | Line speed (mpm / fpm) | 4.3 / 14.1 |  | 2.9 / 9.5 |  | 1.67 / 5.47 | 1.09 / 3.57 |
|  | Rope cap (m / ft) | 22.3 / 73.2 |  | 22.3 / 73.2 |  | 22.1 / 72.5 | 19.5 / 64 |
| 4th layer | Line pull (kg / lb) | 1,628 /3,589 |  | 2,442 / 5,383 |  | 3,446 / 7,599 | 4,097 / 9,033 |
|  | Line speed (mpm / fpm) | 5.0 / 16.4 |  | 3.3 / 10.8 |  | 1.9 / 6.23 | 1.26 / 4.13 |
|  | Rope cap (m / ft) | 24.4 / 80 |  | 30 / 100 |  | 31.9 / 105 | 27 / 89 |
| 5th layer | Line pull (kg / lb) |  |  |  |  | 3,070 / 6,770 |  |
|  | Line speed (mpm / fpm) |  |  |  |  | 2.13 / 6.98 |  |
|  | Rope cap (m / ft) |  |  |  |  | 38 / 125 |  |

Line speed and Amp. Draw

At the first layer of wire rope on the drum.

| ###### Model |  | DV-6000 DV-6000i |  |  | DV-9000 DV-9000i |  |  |  | DV-12000 |  |  |  | DV-15000 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### Line Pull |  | ###### Line Speed |  | ###### Amp. | ###### Line Speed |  | ###### Amp. |  | ###### Line Speed |  | ###### Amp. |  | ###### Line Speed |  | ###### Amp. |  |
| Kg | lb | mpm | fpm | 12V | mpm | fpm | 12V | 24V | mpm | fpm | 12V | 24V | mpm | fpm | 12V | 24V |
| 0 | 0 | 14.6 | 48 | 60 | 13.4 | 44 | 55 | 30 | 8.3 | 27 | 60 | 50 | 6.9 | 22 | 60 | 50 |
| 907 | 2,000 | 7 | 23 | 170 | 4.6 | 15 | 110 | 65 | * | * | * | * | * | * | * | * |
| 1,814 | 4,000 | 4 | 13 | 320 | 3.4 | 11 | 200 | 100 | 2.6 | 8.6 | 200 | 100 | 1.6 | 5.4 | 180 | 90 |
| 2,722 | 6,000 | 3.1 | 10 | 400 | 2.7 | 9 | 230 | 130 | * | * | * | * | * | * | * | * |
| 3,629 | 8,000 | * | * | * | 2.3 | 7.5 | 260 | 170 | 1.7 | 5.8 | 310 | 190 | 1.2 | 3.8 | 280 | 190 |
| 4,082 | 9,000 | * | * | * | 2.0 | 6.4 | 350 | 210 | * | * | * | * | * | * | * | * |
| 4,535 | 10,000 | * | * | * | * | * | * | * | 1.4 | 4.4 | 370 | 230 | 1.0 | 3.3 | 330 | 220 |
| 5,443 | 12,000 | * | * | * | * | * | * | * | 1.2 | 4.0 | 440 | 290 | 0.9 | 2.9 | 385 | 260 |
| 6,805 | 15,000 | * | * | * | * | * | * | * | * | * | * | * | 0.8 | 2.5 | 460 | 340 |

Main Components

How the winch is rated

Load and speed vary according to how

much wire rope is on the drum. The first

layer of rope on the drum delivers the

slowest speed and the maximum

load. A full drum delivers the maximum

speed and the minimum load. For this

reason, winches are rated on their performance

first layer of rope on the drum.

#### Ⅲ. Installation

Before using the winch, make sure all electrical components have no corrosion or damaged; the

environment should be clean and dry. The voltage drop from the battery connections to the winch must not exceed 10% of the nominal voltage under normal operating condition.

Mounting

- It is very important that the winch be mounted on a flat and hard surface in order to make sure the motor, drum and gearbox housing are aligned correctly.

- It is recommended that you use a mounting channel to prevent from

damaging winch or vehicle.

DV-6000 / 9000

12000/15000

DV-6000i/9000i

- It is also recommended that the roller fairlead be fitted to prevent excessive wear of the wire rope.

- Tighten all mounting nuts on motor and roller fairlead securely.

Battery cables connection

Battery cables specification:

| ###### Model |  | ###### DV-6000 | DV-9000 | DV-6000i | ## DV-9000i | DV-12000 | ## DV-15000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Control Type |  | Detachable solenoid pack |  | Integrated solenoid pack |  | Detachable solenoid pack |  |
| Volt | 12V or 24V | Red cable: 2AWG x 1.83m/72” |  | Red cable: 2AWG x 1.83m/72” |  | Red cable: 2AWG x 1.83m/72” |  |
|  |  | Black cable: 2AWG x 1.83m/72” |  | Black cable: 2AWG x 1.83m/72” |  | Black cable: 2AWG x 1.83m/72” |  |

- Attach the black cable (grounding) firmly to the negative (–) battery terminal.

- Attach the red cable to the circuit breaker, connect the other end to the positive (+) battery terminal.

- The circuit breaker must be fitted.

DV-6000/9000                            DV-6000i/9000i

Switch Connection

- A trigger switch withψ1.25mm X 3c X

5m(16AWG X 3c X 17’) cord supplied

Open the dust-proof cover of the winch, then

(Fig3)

insert the switch plug into the socket (fig3).

#### Ⅳ. Operation

### Precautions

Check all safety and environmental conditions prior and during use.

A wire rope should be replaced if it shows signs of excessive wear, broken wires, corrosion or any other defects.

The operator must remain with the winch when it is being operated.

The winches duty rating is S3 (intermittent – periodic)

If the winch fails to pull a load under normal conditions, stop the operation within 30 seconds otherwise motor damage may occur.

Ensure that the winch is connected to the correct voltage. 12VDC or 24VDC only

Check that the clutch lever is in the “Engaged” position during and after use.

Remove the switch from the winch when not in use.

Do not wrap the wire rope around the load and back onto it self. Always use a strap to ensure that the wire rope does not fray or kink.

Keep hands and clothes away from the winch, wire rope, and fairlead.

Never unplug the pendant when winching a load.

Before use, ensure that you are familiar with all winching operations (winch speeds & direction).

To avoid insufficient power when winching a load, the vehicle should be running and in neutral.

Keep the pendant cord clear of the winch cable at all times.

If noise or vibration occurs when running, stop the winch immediately and return it for repair.

### Cable-in/ Cable-out Operation

1). To determine “Cable - Out”, trigger → out  (fig.4)

2). To determine “Cable - In”, trigger ← in  (fig.5)

To stop winching, release the trigger

(Fig4)                             (Fig5)

Free Wheel (Clutch) Function

The clutch allows rapid unspooling of the wire rope for hooking onto a load or anchor points and is operated by a clutch lever.

The clutch lever must be in the “Engaged” position before winching.

1). To disengage the clutch lift the clutch lever up and turn it at 900 clockwise rotation to the “Disengaged” position, wire rope can now be free spooled off the drum.

2). To engage the clutch lift the clutch lever up and turn it at 900 counter-clockwise rotation to the “Engaged” position.

3). If a clutch lever can’t be properly locked in the “Engaged” position, rotate the drum to make the clutch couple to the gear train.

4). Wear leather gloves and use a strap when guiding the wire rope off the drum.

(Engaged)                    (Disengaged)

Recovery Procedures

Followings are some safety tips to get out

of trouble during a recovery.

1).Using a snatch block will increase the capacity, but the speed will decrease accordingly.

2).Experience has shown the best wire rope service is obtained when the maximum fleet angle is not more than 15 degree. Keep the wire rope as close as possible to the centre line of the anchor point.

3).It is the best to work with the most of wire rope unreeled from the drum to get more pulling power, but a minimum of five (5) wraps of wire rope should be wound around the drum to support the rated load.

###### Ⅴ. Maintenance

### Cable Replacement

### Never use a rope of a different size or material and only use genuine

### replacement parts.

### If the winch is subjected to a high duty or excess load, the rope may require frequent replacement.

1). Disengage the clutch (Fig6)

### 2). Spool the entire wire rope, and then remove it from the drum.

### 3). Place the replacement wire rope through the fairlead opening, pass below the

### drum, and insert it into the hole on the drum core. (Fig7)

4). Tighten the screw downwards to secure the wire rope (Fig7).

5). Secure a clevis hook to the wire rope by using a clevis pin and cotter pin.

(Engaged)                    (Disengaged)

(Fig 6)                               (Fig 7)

Brake adjustment
   When the brake wears to the point that the load begins to slip. The brake can be

adjusted as follows:

1). Loosen the bolt on the brake cover and take out c-rings

2). Insert few washers to maintain the brake spacer between to be 2.2 ±0.25 mm

3). Make sure to keep the clutch base plate counter-clockwise by 150 – 180 degree

Lubrication

All moving parts in the winch are permanently lubricated at the time of assembly. Under

normal conditions factory lubrication will suffice. If re-lubrication is necessary after

repair or disassembly use a marine type grease.

.

######  Maintenance Schedule

- Ensure that a responsible person carries out all inspections as per schedule.

- Inspections are dived into Daily, Monthly and 3 Monthly.

| Classification of check |  |  | Item |  | Checking method | Checking reference |
| --- | --- | --- | --- | --- | --- | --- |
| Daily | Periodical |  |  |  |  |  |
|  | One month | Three month |  |  |  |  |
|  |  |  | Installation | Mounting bolts & alignment. | Bolt tension & wear. | Existence of abnormalities |
|  |  |  | Trigger switch | Working | Manual | Reasonable actuation |
|  |  |  |  | Wearing in contact points | Visual. | Free of wear or damage. |
|  |  |  | Wire rope | Broken strands | Visual, measuring (monthly) | Less than 10% |
|  |  |  |  | Decrease in rope diameter | Visual, measuring (monthly) | 7% of nominal diameter max |
|  |  |  |  | Deforming or corrosion | Visual | Existence of abnormalities |
|  |  |  |  | Fastening condition of end | Visual | Existence of abnormalities |
|  |  |  | Clutch | Wear in spring | Visual evidence of wear | Free of wear or damage. |
|  |  |  | Motor | Staining, damage | Visual evidence of wear | Existence of abnormalities |
|  |  |  | Brake | Wearing of lining | Visual evidence of wear | Free of wear or damage. |
|  |  |  |  | Performance | Visual | Reasonable actuation |
|  |  |  | Gear | Damage, wearing | Visual evidence of wear | Free of wear or damage. |

#### Ⅵ. Trouble Shooting

When the winch fails to operate after several attempts, or if there is any fault while operating check the following:

| Symptom | Possible Cause | Remedy |
| --- | --- | --- |
| Winch will not operate | Cut circuit | Check battery cable. |
|  | Weak battery | Recharge or replace battery (at least 650CCA) |
|  | Damaged over load protector | Replace over load protector |
|  | Bad connection of wiring | Reconnect tightly |
|  | Damaged solenoid | Replace solenoid |
|  | Cut circuit on switch | Replace switch |
|  | Damaged motor or worn carbon brush. | Replace motor or carbon brush |
|  | Dropt or lost motor wiring. | Replace wiring or tight it. |
| Motor runs in one direction. | Broken wiring or bad connection | Reconnect or replace wiring |
|  | Damaged or stuck solenoid | Replace solenoid |
|  | Switch inoperative | Replace switch |
|  | Dropt or lost wiring. | Replace wiring and tighten. |
| Drum will not free spool. | Clutch not disengaged | Engaged clutch |
|  | Damaged 1st shaft | Replace 1st shaft |
|  | Damaged brake cam and disc | Replace brake cam and disc |
|  | Damaged output shaft | Replace output shaft |
| No brake | Damaged brake cam and disc | Replace brake cam and disc |
|  | Damaged gear box | Replace gear box |
|  | Dropt snatch ring | Replace snatch ring |
|  | Oil leakage at brake | Clean oil leakage |
|  | Damaged or inoperative spiral spring | Replace and position spiral spring |
| Brake distance is too long | Worn or damaged brake | Replace or adjust brake |
|  | Oil leakage at brake. | Clean oil leakage |
| Brake will be locked | Too much brake powder | Clean brake ass’y |
|  | Over pre-pressed spiral spring | Adjust pre-pressed spiral spring |
|  | Stuck between brake lining and gear box | Replace a new winch |
| Damaged gear box | Hit by certain exterior force. | Replace the damaged components |
|  | Damaged gear train. | Replace the damaged components |
|  | Over load operation. | Replace a new winch |
| Motor runs extremely hot | Long period of operation | Allow to cool |
|  | Damaged motor | Replace or repair motor |
|  | Damaged or inoperative brake | Replace or repair brake |

Ⅶ. Replacement parts List

DV-6000/9000

| No. | Description | Q’ty | No. | Description | Q’ty | No. | Description | Q’ty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Motor | 1 | 23 | Cone brake disc | 1 | 44 | Trigger switch ass’y | 1 |
| 2 | Motor support rack | 1 | 24 | Spiral spring | 1 | 45 | Strap | 1 |
| 3 | Gear support rack | 1 | 25 | Brake cover | 1 | 46 | Snatch block CBV-22 | 1 |
| 4 | Drum bushing | 2 | 26 | Bearing | 1 | 47 | Control box upper cover | 1 |
| 5 | Drum | 1 | 27 | Bearing | 1 | 48 | Control box base cover | 1 |
| 6 | Gear box | 1 | 28 | Bushing | 1 | 49 | Remote switch socket | 1 |
| 7 | Connecting socket | 1 | 29 | Hex. bolt | 6 | 50 | Rubber cover | 1 |
| 8 | 1st shaft | 1 | 30 | Hex. bolt | 3 | 51 | Round cross bolt | 2 |
| 9 | 1st pinion | 1 | 31 | Hex. lockbolt | 1 | 52 | Nut | 2 |
| 10 | 1st stage carrier | 1 | 32 | Hex. bolt | 1 | 53 | Solenoid wiring | 1 |
| 11 | 1st & 2nd ring gear | 1 | 33 | Hex. bolt | 1 | 54 | Grounding cable | 1 |
| 12 | 2nd stage carrier | 1 | 34 | Fixed bolt | 6 | 55 | Tie bar | 2 |
| 13 | 3rd stage carrier | 1 | 35 | C-ring | 2 | 56 | Hex. Nut` | 4 |
| 14 | 3rd ring gear | 1 | 36 | C-ring | 2 | 57 | Spring washer | 4 |
| 15 | 3rd washer | 1 | 37 | Gasket | 2 | 58 | Cross bolt | 5 |
| 16 | Clutch sleeve | 1 | 38 | Spring pin | 1 | 59 | Dented washer | 5 |
| 17 | Clutch lever | 1 | 39 | Mounting board | 1 | 60 | Bolt | 2 |
| 18 | Compression spring | 1 | 40 | Wire rope | 1 | 61 | Nut | 4 |
| 19 | Clutch knob | 1 | 41 | Roller fairlead | 1 | 62 | Spring washer | 2 |
| 20 | Brake clutch base | 1 | 42 | Mounting hardware | 1 |  |  |  |
| 21 | Brake cam | 1 | 43 | Clevis Hook | 1 |  |  |  |

DV-6000i/9000i

| No. | Description | Q’ty | No. | Description | Q’ty | No. | Description | Q’ty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Motor | 1 | 23 | Cone brake disc | 1 | 44 | Trigger switch ass’y | 1 |
| 2 | Motor support rack | 1 | 24 | Spiral spring | 1 | 45 | Strap | 1 |
| 3 | Gear support rack | 1 | 25 | Brake cover | 1 | 46 | Snatch block CBV-22 | 1 |
| 4 | Drum bushing | 2 | 26 | Bearing | 1 | 47 | Control box left cover | 1 |
| 5 | Drum | 1 | 27 | Bearing | 1 | 48 | Control box right cover | 1 |
| 6 | Gear box | 1 | 28 | Bushing | 1 | 49 | Empty control box | 1 |
| 7 | Connecting socket | 1 | 29 | Hex. bolt | 6 | 50 | Rubber Clamp | 1 |
| 8 | 1st shaft | 1 | 30 | Hex. bolt | 3 | 51 | Hex. bolt | 6 |
| 9 | 1st pinion | 1 | 31 | Hex. lockbolt | 1 | 52 | Hex. bolt | 4 |
| 10 | 1st stage carrier | 1 | 32 | Hex. bolt | 1 | 53 | Spring washer | 4 |
| 11 | 1st & 2nd ring gear | 1 | 33 | Hex. bolt | 1 | 54 | Solenoid wiring | 1 |
| 12 | 2nd stage carrier | 1 | 34 | Fixed bolt | 6 | 55 | Grounding cable | 1 |
| 13 | 3rd stage carrier | 1 | 35 | C-ring | 2 | 56 | Trigger switch socket | 1 |
| 14 | 3rd ring gear | 1 | 36 | C-ring | 2 | 57 | Cross bolt | 2 |
| 15 | 3rd washer | 1 | 37 | Gasket | 2 | 58 | Nut | 2 |
| 16 | Clutch sleeve | 1 | 38 | Spring pin | 1 | 59 | Rubber cover | 1 |
| 17 | Clutch lever | 1 | 39 | Mounting board | 1 |  |  |  |
| 18 | Compression spring | 1 | 40 | Wire rope | 1 |  |  |  |
| 19 | Clutch knob | 1 | 41 | Roller fairlead | 1 |  |  |  |
| 20 | Brake clutch base | 1 | 42 | Mounting hardware | 1 |  |  |  |
| 21 | Brake cam | 1 | 43 | Clevis hook | 1 |  |  |  |

DV-12000/15000

| No. | Description | Q’ty | No. | Description | Q’ty | No. | Description | Q’ty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Motor | 1 | 25 | Cone brake disc | 1 | 49 | Spring packing | 9 |
| 2 | Motor support rack | 1 | 26 | Brake base | 1 | 50 | Mounting hardware | 1 |
| 3 | Gear support rack | 1 | 27 | Brake clutch base | 1 | 51 | Roller fairlead | 1 |
| 4 | Drum bushing | 2 | 28 | Brake cover | 1 | 52 | Trigger switch ass’y | 1 |
| 5 | Drum | 1 | 29 | Fix lever | 2 | 53 | Solenoid pack | 1 |
| 6 | Socket | 1 | 31 | C-ring | 1 | -1 | Up cover | 1 |
| 7 | 1st shaft | 1 | 32 | Strap | 1 | -2 | Down cover | 1 |
| 8 | 1st shaft gear | 1 | 33 | Wire rope | 1 | -3 | Wiring ass’y | 1 |
| 9 | 1st pinion | 1 | 34 | Bearing | 2 | -4 | Gland | 1 |
| 10 | 2nd stage carrier | 1 | 35 | Gasket A | 2 | -5 | Cross screw | 2 |
| 11 | 3rd stage carrier | 1 | 36 | Gasket B | 1 | -6 | net | 2 |
| 12 | 1st & 2nd ring gear | 1 | 37 | C-ring | 2 | -7 | Solenoid ass’y | 1 |
| 13 | 3rd ring gear | 1 | 38 | Hex. bolt | 4 | -8 | Hex. bolt | 5 |
| 14 | Gear box | 1 | 39 | Hex. bolt | 10 | -9 | Dented washer | 5 |
| 15 | Plain washer | 1 | 40 | Hex. bolt | 9 | -10 | Hex. bolt | 2 |
| 16 | Clutch sleeve | 1 | 41 | Hex. bolt | 1 | -11 | Nut | 4 |
| 18 | Compression spring | 1 | 42 | Hex. bolt | 1 | -12 | Spring packing | 2 |
| 19 | Clutch knob | 1 | 43 | Hex. bolt | 1 | 54 | Mounting board | 1 |
| 20 | Spring pin | 1 | 45 | Hex. bolt | 10 | 55 | Clevis hook | 1 |
| 21 | Clutch knob | 1 | 46 | Spring packing | 10 | 56 | Grounding cable | 1 |
| 22 | Spiral spring | 1 | 47 | Spring packing | 4 | 57 | C-ring | 1 |
| 24 | Brake cam | 1 | 48 | Spring packing | 10 |  |  |  |

Limited Warranty

This Limited Warranty is given by the Chuan Phang Ent. Co., Ltd. (the “Seller”) to the original purchaser (the “Purchaser”) of a  Winch specified in this manual. This Limited Warranty is not transferable to any other party.

The Seller takes the responsibility for all parts and components, with the exception of the wire rope, to be free from defects in materials and workmanship appearing under normal use for as long as the said Purchaser owns the vehicle that the winch was originally mounted on. Electrical components are warranted for 1 Year from date of purchase under the same conditions. Any  Winch, which is defective, will be repaired or replaced without charge to the Purchaser.

Upon discovering any defect, the Purchaser under this Limited Warranty is requested to return the complete winch and inform the seller or their authorised distributors of any claims. The Purchaser must provide a copy of the proof of purchase bearing the winch serial number, date of purchase, owners name and address, vehicle details and registration number.

The Limited Warranty does not cover any failure that results from improper installation, operation or the Purchaser’s modification in design. The winch is designed for vehicle self-recovery use only and should not be used in industrial applications or for moving people. The Seller does not warrant them to be suitable for such use.